Looking for Thermal Inefficiencies in Your Classroom

Materials to Prepare:

- Map of classroom

- Pack of sticky notes

- Clipboards

- Thermal imaging device

Academic vocabulary:

- Radiation

- Convection

- Conduction

- Insulation

General outline of the lesson:

- Review how heat moves- opening questions:

  - Where do you think the warmest place is in the room? Why?

  - Where do you think the coldest place in the room is? Why?

- Prompt/discuss the importance of keeping buildings comfortable (cool in summer, warm in winter).

  - What do you know about how buildings are constructed and what materials are used?

  - What steps do people take to regulate the temperature in a room or building?

- Students make predictions about where thermal inefficiencies are in the classroom.

  - Students, individually or in groups, mark their predictions on a map using a pencil.

  - Then students place sticky notes on the locations in the room.

- Review with the students how to use the thermal scanner.

- Do assessment of thermal energy from inside the classroom-

- Crank up heat and look for leaks from outside of the room.

  - best to do early in the day when it is likely to be cooler outside.

- Mark on classroom map with a pen, colored pencil, or marker where heat loss was observed.

6. Check for Understanding- Exit ticket

Have students answer the following two questions on a slip of paper and submit to teacher.

- How is heat lost from a structure?

- What factors can affect the loss of heat from a structure?