12-Sided Solar Cooker

Websites for more help:

http://solarcooking.org/plans/dats.htm

https://www.youtube.com/watch?v=YVGUexfEPFs

https://www.youtube.com/watch?v=h2JKVzz6cRs

Ingredients for making a 12-sided solar cooker:

- material for panels - tag board/old but sturdy folders (might be too flimsy), card board,  OR 2–3 science fair boards

- thick string for tying the panels together

- aluminum foil (heavy duty works the best because it’s sturdier)

- a stick for tightening the string

- packing tape (heavy duty works best)

Directions:

- Using the diagram below and the attached table, scale down the panel to half the size of the original.

- Using a ruler, draw a new panel onto the tag board I gave you. Label the template you made with your new measurements to look like the sample I gave you on the back of these directions. Cut it out and use it to draw 12 panels onto the material you decided to use to make your panels.

- Cut out the 12 panels and use a ruler to draw the dotted lines where they go.

- Make a fold on the dotted lines for each panel.

- Cover one side of each panel with aluminum foil – make sure it overlaps the edges so that it stays on.

- Tape the panels together with the packing tape – aluminum foil side is the inside.

- Punch two holes where shown on the sample given.

- Fold the ears (flaps) away from you and string the panels together.

- Use the string to weave in and out and attach the panels together.

| Original Shape | Original Size | Scaled Down Size Scale Factor | Original Size | Scaled Down Size Scale Factor | Original Size | Scaled Down Size Scale Factor |
| --- | --- | --- | --- | --- | --- | --- |
|  | 23 cm |  | 37 cm |  | 13 cm |  |
|  | 24 cm |  | 3.5 cm |  |  |  |
|  | 11.5 cm |  | 5 cm |  |  |  |